1. HC-SR04 Ultrasonic Sensor

- Trig Pin: Connect to Arduino digital pin 9.

- Echo Pin: Connect to Arduino digital pin 10.

- VCC: Connect to Arduino 5V.

- GND: Connect to Arduino GND.

2. L298N Motor Driver

- Motor A (Left Motor):

  - ENA (Enable A): Connect to Arduino PWM pin 5 for speed control.

  - IN1: Connect to Arduino digital pin 6.

  - IN2: Connect to Arduino digital pin 7.

  - Motor Output Terminals:

    - OUT1: Connect to the positive terminal of the first motor.

    - OUT2: Connect to the negative terminal of the first motor.

- Motor B (Right Motor):

  - ENB (Enable B): Connect to Arduino PWM pin 11 for speed control.

  - IN3: Connect to Arduino digital pin 8.

  - IN4: Connect to Arduino digital pin 12.

  - Motor Output Terminals:

    - OUT3: Connect to the positive terminal of the second motor.

    - OUT4: Connect to the negative terminal of the second motor.

- 12V Motor Power Supply:

  - Connect the positive terminal of a 12V power supply to the +12V terminal on the L298N.

  - Connect the negative terminal of the 12V power supply to the GND terminal on the L298N.

- Ground Connection: Connect the GND on the L298N to the GND of the Arduino to ensure a common ground.

3. Potentiometer (For Manual Speed Control)

- Middle Pin (Wiper): Connect to Arduino analog pin A0.

- One Side Pin: Connect to Arduino 5V.

- Other Side Pin: Connect to Arduino GND.

4. 16x2 I2C LCD Display

- SDA (Serial Data): Connect to Arduino A4.

- SCL (Serial Clock): Connect to Arduino A5.

- VCC: Connect to Arduino 5V.

- GND: Connect to Arduino GND.

5. Arduino Power Supply

- VIN Pin: If you're using the 12V motor power supply for the entire system, connect the 12V power supply positive terminal to the Arduino’s VIN pin.

- GND: Connect the GND of the Arduino to the shared ground with the L298N and the 12V power supply.

Summary of Pin Connections to Arduino

- HC-SR04 Sensor: Trig (Pin 9), Echo (Pin 10)

- Motor A: ENA (Pin 5), IN1 (Pin 6), IN2 (Pin 7)

- Motor B: ENB (Pin 11), IN3 (Pin 8), IN4 (Pin 12)

- Potentiometer: Middle pin (A0)

- I2C LCD: SDA (A4), SCL (A5)

Power Supply Notes

- Use the 12V power supply to power the motors via the L298N.

- The same 12V power supply can power the Arduino through the VIN pin, and the Arduino will regulate it to 5V for its internal components.